PARKINSONSAT TELEMETRY DEFINITIONS AND EQUATIONS    Rev-1   14 Feb 2007

--------------------------------------------------------------------

The 20 channels of PARKINSONSAT telemetry are multiplexed into 4 sequential frames.  Each frame consists of 5 analog channels, and 8 status bits which include the two FRAME counter bits.  It takes 4 frames to constitute the complete telemetry picture of PARKINSONSAT basic health and status.  Frame Format:

PSAT>BEACON,SGATE:T#SSS,111,222,333,444,555,FF111111

Where the header is PSAT>BEACON,SGATE:

Within the actual data portion of the packet are these fields:

SSS      is the serial number

111      is the first channel telemetry

222      is the second channel

333      is the third channel

444      is the forth channel

555      is the fifth channel

FF111111 the 8 internal bits (including the FRAME counter)

A-SIDE

FF  CHANNEL 1 	CHANNEL 2  	CHANNEL 3  	CHANNEL 4  	CHANNEL 5

--  --------- 	---------- 	---------- 	---------- 	----------

11  SOLAR+X-Amps	SOLAR-X-Amps SOLAR+Y-Amps 	SOLAR-Y-Amps	SOLAR+Z-Amps

10  TNC/RX-Amps	ODTML-Amps 	ADCS-Amps 	RFI-Amps 		MiDN-Amps

01  TX-Temp	Z-Temp    	-Z-Temp    	+X-Temp    	BATT-Temp

00  8BUS-Volt 		14BUS-Volt   	SOLAR-Amps 	BAT-Amps 		LOAD-Amps

Define Function for Temperature:

Tof(V) = .00001 * X ^ 3 - .0039 * X ^ 2 + .829 * X - 40.4

IF Frame is "11" THEN

SOLAR+X-Amps = TBD

SOLAR-X-Amps = TBD

SOLAR+Y-Amps = TBD

SOLAR-Y-Amps = TBD

SOLAR+Z-Amps = TBD

IF Frame is "10" THEN

TNC/RX-Amps = TBD

ODTML-Amps  = TBD

ADCS-Amps   = TBD

RFI-Amps    = TBD

MiDN-Amps   = TBD

IF Frame is "01" THEN

TX-Temp     = TBD

+Z-Temp     = TBD

-Z-Temp     = TBD

+X-Temp     = TBD

BATT-Temp     = TBD

IF Frame is "11" THEN

8BUS-Volt   = X1 * 0.1

14BUS-Volt  = X1 * 0.1

SOLAR-Amp   = TBD

BAT-Amps    = TBD

LOAD-Amps   = TBD

B-SIDE

FF  CHANNEL 1 	CHANNEL 2  	CHANNEL 3  	CHANNEL 4  	CHANNEL 5

--  --------- 	---------- 	---------- 	---------- 	----------

11  SOLAR+X-Amps	SOLAR-X-Amps SOLAR+Y-Amps 	SOLAR-Y-Amps	SOLAR+Z-Amps

10  TNC/RX-Amps	ODTML-Amps 	ADCS-Amps 	RFI-Amps 		MiDN-Amps

01  TX-Temp 	+Z-Temp    	-Z-Temp    	-X-Temp    	BATT-Temp

00  8BUS-Volt 		14BUS-Volt   	SOLAR-Amps 	BAT-Amps 		LOAD-Amps

Telemetry equations are generally of the form y = aX + b

where X is the 3 byte decimal telemetry count in each channel

in the data. Temperature is a cubic y=aX^3 + bX^2 + cX - d.

Define Function for Temperature:

Tof(V) = .00001 * X ^ 3 - .0039 * X ^ 2 + .829 * X - 40.4

IF Frame is "11" THEN

SOLAR+X-Amps = TBD

SOLAR-X-Amps = TBD

SOLAR+Y-Amps = TBD

SOLAR-Y-Amps = TBD

SOLAR+Z-Amps = TBD

IF Frame is "10" THEN

TNC/RX-Amps = TBD

ODTML-Amps  = TBD

ADCS-Amps   = TBD

RFI-Amps    = TBD

MiDN-Amps   = TBD

IF Frame is "01" THEN

TX-Temp     = TBD

+Z-Temp     = TBD

-Z-Temp     = TBD

-X-Temp     = TBD

BATT-Temp     = TBD

IF Frame is "11" THEN

8BUS-Volt   = X1 * 0.1

14BUS-Volt  = X1 * 0.1

SOLAR-Amp   = TBD

BAT-Amps    = TBD

LOAD-Amps   = TBD